EET 200

# PROTEL SIMULATION LABORATORY

09/24/03

GOAL:

- To learn the basics of schematic simulation with Protel.

ASSIGNMENTS:

- Multivibrator (with instructor)

- Push-Pull Amplifier (submit to Dan Kohn)

# MULTIVIBRATOR

PARTS:	R1, R2, R3, R4 	RES		*NOTE: must only use Sim Library!

C2, C2 		CAP

Q1, Q2 		2N3904

V1			+12V source

General Simulation Steps:

1st:	Draw schematic and label all attributes.

2nd:	Assign appropriate net labels for measuring.

3rd:	Run ERC

4th:	Simulate  Setup…

5th	Setup for appropriate analysis; choose active signals

6th:	Run Analysis

7th:	Set up waveforms with appropriate scaling, cursors, and designation symbols

# PUSH-PULL AMPLIFIER

PARTS:	R1 = 100k		*Remember to only use Sim Library!

R2 = 10k

R3 = 10k

R4 = 100k

RL = 200

C1 = 1.26u

Q1 = 2N2219A, Q2 = 2N2905A

Net Labels: VCC, VSS, IN, OUT, A, B1, B2

CON2 = VSIN (Sinusoidal Voltage Source) @1 (AC), 1 (Amplitude), 1k (freq.)

BT1 =  +12V Simulation Source (Label it VCC)

BT2 =   -12V Simulation Source (Label it VSS)

Lab requirements:

- Name DDB as:  Your Initials-Sim.ddb

- Name the schematic file as Push-Pull.sch

- Schematic drawn and labeled appropriately with properly labeled Title Block

- ERC run

- Simulation Setup as follows:

Transient/Fourier Analysis - ON

    - CALCULATE Stop time, Start time, and Max Step

    - Stop Time = 1 / lowest frequency source * Cycles Displayed

    - Step Time = (1/ lowest frequency source) / Points per Cycle

    - Max Step = Step Time

AC Small Signal – ON

- Start Frequency – 1kHz

- Stop Frequency – 10kHz

- Test points – 50

Operating Point Analysis – ON

Collect Data For:	Node Voltage, Supply and Device Current

Active Signals: 	B1, B2, IN, OUT

- Run Analysis

- For Transient Analysis: View B1, B2, IN, OUT as a single cell with designation symbols active

- For AC Analysis: View B1, B2, IN, OUT as separate cells with data points active

- For Transient Analysis:  Place measurement cursors for IN at 0.500ms; OUT at 1.000ms.

- For AC Analysis:  Place measurement cursors for IN at 2kHz; OUT at 4kHz.

- For Operating Point:  Display B1, B2, IN, OUT

*Submit DDB file to Dan Kohn.  (http://ceetusm.com/~EET200/)